Paleomagnetic Analysis Program V3.2

Handbook

Overview

Paleomagnetic Analysis Program V3.2 is meant to aid in the interpretation and publication of paleomagnetic data.  It can be used to create Zijderveld and Stereographic or Lambert equal-area plots for a sample, run analysis routines on the samples, and print graphic output.

Paleomagnetic Analysis Program V3.2 was completely redesigned in December 2000 based on a previous PALEOMAG program, which was developed in the late 1980’s at Purdue University prior to the age of Microsoft Windows.  The former PALEOMAG program developed its own version of a Windows system, which causes some incompatibility with the later Microsoft product. Therefore the PALEOMAG program can only and must be run under DOS. The new Paleomagnetic Analysis Program V3.2 is a windows-based program with graphical user interface, and is much easier to use. It corrected some bugs in the previous PALEOMAG program, and also added some functionalities.

This handbook assumes that the user has the basic knowledge of Microsoft Windows operating system and knows how to use a mouse.

General Procedure of Paleomagnetic Data Analysis

- Start Paleomagnetic Analysis Program V3.2

- Convert *.app or open *.mag data files

- Visualize the paleomagnetic data

- Statistical analysis of paleomagnetic data

- Printing graphic outputs

- Start Paleomagnetic Analysis Program V3.2

Since you are running Virtual PC on Macintosh whose mouse has only one button, just click Start > Programs > Paleomagnetic Analysis Program using the mouse to start the program.

- Convert *.app or open *.mag data files

The first step in paleomagnetic data analysis is usually converting a *.app file – a fixed-length ASCII file storing the paleomagnetic data to a random-access file which is used by the Paleomagnetic Analysis Program. For some unknown reason(s) however, the Paleomagnetic Analysis Program cannot convert *.app data files in the Virtual PC environment. Therefore, the already converted and ready to use data files (*.mag) are provided for your use in the Magnetostratigraphy Lab of GEOS 474. If you would like to run the program on a computer with Microsoft Windows 95 or later, just copy the paleomag.exe file located in the C:\Program Files\ PaleomagneticAnalysis folder to that computer. All the functions of the Paleomagnetic Analysis Program should work without any problem.

To open a data file (*.mag), click the menu File > Open (or press the combination key Ctrl + O). A dialog box “Open *.mag File” pops up. Select the appropriate drive by clicking the drop-down box in the upper-left corner in the File Selection frame. Change the directory/folder by double-clicking a folder in the lower left folder-list box. If there is any *.mag file in the folder you just double-clicked, it will be listed in the small window on the right. For this lab, just double click the Magstrat Lab folder and a list of *.mag files will appear in the right-hand-side window. Select the desired file by clicking on the file name and then click Open or by double-clicking the filename. A list of samples will appear in the left window (refer to hereafter as the Sample Window) on the main window of the program.

- Visualize the paleomagnetic data

Click a sample listed in the Sample Window will cause the data of that sample to be displayed in the right-hand-side window (refer to hereafter as the Data Window) on the main window. The abbreviations used in the Data Window and their meaning are as follows:

  - Steps: the demagnetization steps. NRM = Normal Remanence Magnetism, TT = Thermal Demagnetization, ORG = Origin. The numbers following TT in the Steps column are the temperatures (in C) at which the demagnetization were carried out;

  - GDec and GInc: geographic declination and inclination (in degrees);

  - TDec and TInc: tilt-corrected declination and inclination (in degrees);

  - Intensity: intensity of the paleomagnetic field ( in 10-3 Am-1);

  - Error: a relative analytical error for the measured magnetic vector.

  - LSquare: indicates if a demagnetization step is used the least-squares analysis. The default is “No”.

  - Plot: indicates if the data of a demagnetization step will be used in the various projections, e.g., Zijderveld-, stereographic-, and Lambert equal-area projection. The default is “Yes”.  As the origin (ORG) is not actually a data point, it will not be used in the projections.

There are several ways to visualize the demagnetization data of a sample. These methods are as follows:

  - Zijderveld projection: The “Zijderveld” vector display separates the declination-intensity and the inclination-intensity components of the magnetic vectors. The graphic in the Paleomagnetic Analysis Program automatically focuses on the portion of the polar-coordinate grid that contains the sample data, in order to display the Zijderveld diagram as large as possible. The superimposed axes for declination (inclination) are E-W (horizontal) and N-S (up-down), and the scaling of tick marks on the axes is indicated at the top of the plot. If the true inclination is displayed without projection onto a fixed plane, then the phrase “No Projection” is shown. However, the user may project the inclination onto a fixed vertical plane by simply selecting the corresponding menu in the main window and entering the declination of the desired plane (0-360). To perform Zijderveld projection: select a sample, then click the menu Zijderveld > Geographic (or Tilt-Corrected if you want to use the tilt-corrected data), and the diagram will be shown in the left panel of the projection window. After this step, simply click any sample you want in the main window, and the Zijderveld projection of that sample will be automatically displayed in the projection window.

  - Stereographic and Lambert Equal-Area Projections: These projections share the right panel of the projection window. Stereographic and Lambert equal-area projections of vector arrays sacrifice the intensity component to enable easy visualization of directional trends with progressive demagnetization. In the Paleomagnetic Analysis Program, the progressive steps are connected with the initial NRM direction plotted as a blue square symbol and the other demagnetization steps as circles. A filled circle indicates a downward (positive) inclination, while a hollow circle represents an upward (negative) inclination. Red-colored symbols indicate that the demagnetization steps have not been selected for use in the least-squares analysis of characteristic directions, while green-colored symbols denote steps used in the least-squares analysis (described below). To perform Stereographic  projection: select a sample, then click the menu StereoPlot > Geographic (or Tilt-Corrected if you want to use the tilt-corrected data), and the diagram will be shown in the right panel of the projection window. To perform Lambert Equal-Area  projection: select a sample, then click the menu EQAreaPlot > Geographic (or Tilt-Corrected if you want to use the tilt-corrected data).

  - Displaying the Entire Suite of Sample Directions: It is desirable to view the directional data from the entire suite of samples at a selected demagnetization step to examine general trends and scatter. Plots of the characteristic directions derived from the sample-by-sample least-squares analysis are valuable in visualizing polarity clustering before performing Fisher statistics to derive mean directions. To Display the Entire Suite of Sample Directions: click the menu All > Stereo Plot All > NRM Data etc., and the diagram will be shown in the right panel of the projection window. The default projection is Lambert equal-area projection using tilt-corrected data. But you can perform any projection (Stereographic or Lambert equal-area) using any data (Geographic or Tilt-corrected) by first performing the desired projection using the desired data on a single sample.

- Statistical analysis of paleomagnetic data

This includes the least-squares vector analysis of individual samples and the Fisher analysis of mean directions of the entire section.

  - Least-Squares Vector Analysis: The user can click in the check-box(es) in front of the desired demagnetization step(s) in the main window, and then click the “Select/Remove Points for Least Square Analysis” command button on the main window to select the step(s) for least-squares analysis. A dialog box comes up prompting you to enter a polarity designation for the sample. Follow the instruction to enter the appropriate polarity designation. The least-squares analysis will be automatically performed, and the result will be displayed the main window right below the sample data. The polarity designations are as follows:

    - N or R: for samples with obvious Normal or Reversed polarity

    - NP or RP: for samples with fairly certain Normal or Reversed polarity.

    - NPP or RPP: for samples with barely recognizable Normal or Reversed polarity.

    - N? or R?: for samples with questionable polarity

    - INT: for samples with polarity close to the East or West direction

The user can view the results of least-squares analysis for all the samples in a separate “Least Squares Analysis Results”. The lower portion of this listing has a table of the demagnetization steps chosen for the least-squares analysis of each sample.

  - Fisher Analysis of Mean Directions: Fisher statistics of mean directions can be done using either geographic or tilt-corrected data of any designated demagnetization step or the suites of directions and associated polarities from least-squares analysis. Mean directions are computed for both the normal and the reversed polarities, for the entire set after inverting reversed-polarity vectors to combine with normal-polarity vectors, and from addition of the mean vectors from the two polarities. A log-mean intensity is computed for each set as a representative relative measure of the characteristic magnetization vector. Because mean directions are computed for each polarity cluster, a general estimate of a typical declination and inclination corresponding to normal polarity is required for Fisher analysis of NRM or a single demagnetization step. This estimate is not required when using characteristic directions in which a polarity label has been assigned to each sample.

There is an option to “weight” the directions computed by the least-squares procedure. We often incorporate a “reliability rating” in our polarity designator during the least-squares procedure, in which excellent results are indicated by “N” or “R” and less precise data are assigned “NP” or “RP”. Noisy data or uncertain polarity interpretations assigned other codes (“N?”, “INT”, “RPP”, etc.) are automatically omitted by the program from the mean directions. If the user selects the data to be weighted, then the samples designated  “NP” or “RP” are given half weight relative to “N” and “R”.

Two iterations of Fisher statistics are performed for each polarity cluster. The second iteration omits outliers outside an angular dispersion from the mean direction of each polarity, with the cutoff of the dispersion set as the lesser of 60 or twice the standard deviation given by the Fisher statistics obtained in the first iteration. The output lists the samples omitted from Fisher analysis, either because their polarity designations indicated that they should be omitted or because their directions were considered outliers.

After each iteration, a paleolatitude and projected magnetic pole (latitude, longitude, error) are computed for each polarity and for the entire sample set.

To perform Fisher Analysis: select menu All > Fisher statistics > NRM Data etc., then follow the instructions to select the desired data set, input the estimated inclination and declination, and the option to “weigh” or not to “weigh” the sample as appropriate. Note: If you don’t want to include a sample in Fisher analysis, check mark that sample in the Sample Window, then click the (De)activate Record command button in the main window, and the “Active” indicator of that sample will be changed to “No”. By default, every sample is set to active. A sample whose “Active” status was changed to “No” can be changed back to “Yes” by check-marking that sample and click the (De)activate Record button again.

- Printing graphic outputs

By selecting the menu File > Print > Main Data Window etc., the user can output color or black & white prints of the main window (or any graphic projection) by the Paleomagnetic Analysis Program. By printing “to file” or using screen capture, it is easy to load the output into other graphic software, e.g. Adobe Illustrator, to add additional highlights for publication.

For More Information

The user can use the help menu (or press F1) to get information about the format of the data file and the menu system of the Paleomagnetic Analysis Program. The contact information of the author is provided in the Help > About Paleomagnetic Analysis Program (or press F2).

The user is also encouraged to read the two files located in the C:\Program Files\PaleomagneticAnalysis\User's Guide folder – New User's Manual.doc and PaleomagHelp.doc. The former provides information about system requirement, installation, and un-installation etc. of the program, and also has abundant screen captures. The latter contains the same information as in the help menu.

Good Luck and Enjoy!